This document contains sii3114ide-specific documentation information.

Please see also :

- ReleaseNotes/sii3114ide_dev_relnotes

How to use

==========

- To use the driver, just drop a sii3114-based SATA board

into any PCI slot, connect it to a SATA drive (a real one,

not a PATA with a SATA converter, if you want a real SATA

setup), and add the driver to your kicklayout.

- This driver can work happily aside a1ide and sii0680ide

and sii3112ide.

- You cannot boot yet directly from the sii3114 from within

UBoot, this will come later.

- Due to the inner working of the sii3114, the SATA port

to unit mapping is like this:

o port 0 -> unit 0 (or port 1 if your board counts ports from 1)

o port 1 -> unit 2 (or port 2 ...)

o port 2 -> unit 1 (or port 3 ...)

o port 3 -> unit 3 (or port 4 ...)

- For now, the driver uses two tasks (and not four, while

it is possible to have all 4 channels transfer in

parallel throgh the use of memory mapped registers, but

this is not reliable yet). The tasks handle the units like this:

o port 0 task handles units 0 and 1 (ie ports 0 and 2)

o port 1 task handles units 2 and 3 (ie ports 1 and 3)

This means, if you have 2 SATA drives and want to operate them

in parallel, connect them like this :

o drive 1 on port 0 and drive 2 on port 2 (if your board

counts ports from 1), or

o drive 1 on port 1 and drive 2 on port 3

From version 51.49 on, the driver supports up to 4 PCI chips / boards.

I - Configuring sii3114ide through UBOOT envvars

================================================

Currently sii3114ide is configurable through the following UBOOT envvars :

- sii3114ide_verbose

Legal values are : set to anything makes the driver verbose at boot time.

Default if unset : driver silent.

- sii3114ide_maxbus

Legal values are :  0 (no IDE interface at all),

1 (only primary IDE interface), and

2 (both IDE interfaces)

Default : if you don't set this envvar, sii3114ide will use both IDE interfaces.

- sii3114ide_timeout

Legal values are between 1 and 30 (seconds). The latter is the recommended

value from the ATA(PI) spec.

Default : if you don't set this envvar, sii3114ide will use a 20 second timeout.

- sii3114ide_conf

Compose 4 chars (prim master, prim slave, sec master, sec slave) using :

0 : nothing

1 : harddisk

2 : dvd/cdrom reader or burner

Default : if you don't set this envvar, sii3114ide will scan the IDE buses and

try to determine the configuration by itself.

- sii3114ide_xfer

Allows to define the xfer mode which will be used for each drive. Compose

4 chars (prim master, prim slave, sec master, sec slave) using :

0 - Automatic PIO (driver will use the best PIO mode

that the drive claims to support)

a - PIO 0  (  3 MB/s, modeid 8)

b - PIO 1  (  5 MB/s, modeid 9)

c - PIO 2  (  8 MB/s, modeid 10)

d - PIO 3  ( 11 MB/s, modeid 11)

e - PIO 4  ( 16 MB/s, modeid 12)

A - UDMA 0 ( 16 MB/s, modeid 64)

B - UDMA 1 ( 25 MB/s, modeid 65)

C - UDMA 2 ( 33 MB/s, modeid 66)

D - UDMA 3 ( 44 MB/s, modeid 67)

E - UDMA 4 ( 66 MB/s, modeid 68)

F - UDMA 5 (100 MB/s, modeid 69)

G - UDMA 6 (133 MB/s, modeid 70)

NB : sii3114ide will fall back to the best mode that the drive claims to support

in case you ask for an unsupported mode.

Default : if you don't set this envvar, sii3114ide will use the best PIO mode

supported by the drive.

- sii3114ide_irq

Compose 4 chars (prim master, prim slave, sec master, sec slave) using :

1 : use IRQs

any other char : don't use IRQs

Default : if you don't set this envvar, sii3114ide WILL use interrupts.

Using interrupts relieve the CPU in all cases (PIO : approx 20%, and UDMA

approx 95%), but it also has an incidence on xfer speed : 10 to 20% lower

for PIO, 10 to 20% faster for UDMA. This is normal (in PIO, it involves 2

exec reschedulings, and in UDMA, it releases the VIA bus).

EXAMPLE : to use only the primary interface, set ide timeout to 5 seconds,

force prim master as harddisk, sec master as cd, nothing for other

drives, force prim master to udma33 and let sii3114ide select the xfer

mode for other drives, enter the following commands at the UBOOT

prompt :

setenv sii3114ide_maxbus 1

setenv sii3114ide_timeout 5

setenv sii3114ide_conf 1020

setenv sii3114ide_xfer C000

saveenv

Then reboot. Don't forget the 'saveenv', as sii3114ide uses nonvolatile.library,

which only accesses the envvars stored in nvram.

The driver supports up to 4 sii3114ide PCI boards / chips. For the 2nd, 3rd etc.

chips, add '.2', '.3' etc. to envvar names. I.e., setenv sii3114ide_conf.2 1020

II - Notes

==========

- Since 51.48, this driver runs one task per IDE channel (or port

or chain or interface),  i.e. one task for the primary IDE chain

and one task for the secondary IDE chain. Those tasks operate in

parallel, which means that the driver can write to a CD on the

secondary channel while reading from a harddisk on the primary

channel. So from this version on, it makes sense to dispatch your

IDE peripherals accordingly.

- This configurability feature relies on two kickstart modules from Ross

Vumbaca that MUST be listed in your kicklayout : nonvolatile.library.kmod

50.2+ and nvram.resource.kmod. If those modules are not present, sii3114ide will

revert to default values.

- These envvars HAVE to be saved ('saveenv' after modifying them), because

they are read directly from the nvram.

- To delete an envvar, just define a blank content & then saveenv (ie

'setenv sii3114ide_conf' and then 'saveenv' will delete this envvar).

- Using sii3114ide_conf (properly set !) saves need for timeouts for drive

detection at boot time.

- It looks like each CD/CDR/DVD/DVDR drive reports its own status codes

when a blank media is inserted. Thus, OS4 will report different info

on different systems when a blank CDR is inserted.

- Colin Wenzel reports that the IDE ZIP works.

- When you grab a boot log & send it to me, please use a nondebug kernel.

- If you experience VERY SLOW harddisk writes with FFS, then you probably

have a drive that does not execute properly a FLUSH_CACHE command

(sii3114ide checks that at init time). Thus, sii3114ide disables the drive's

write cache. Thus, your drive really crawls in writing.

This does not show for the same drive with other drivers under other

os'es, because they don't check if the drive can execute a FLUSH_CACHE,

and just leave the write cache ON.

Under Linux or AOS3.x, this can *look* fast & reliable, but this *is* fast

and not reliable.

Some filesystems depend on proper execution of FLUSH_CACHE, when *they*

decide it, not when the drive decides. Thus, those fs'es can appear to be

working, but they are not really reliable. SFS is one of those.

Thus, it was decided (long ago) by the os4 dev team (mainly js & myself ;),

to disable the write cache if the drive cannot properly execute a

FLUSH_CACHE.

The consequence is that under those circumstances, FFS really crawls at

write, but you can move to the latest SFS + its diskcache.library,

which works around this problem while (mostly) staying on the safe side.

Make sure to create your SFS partition this way :

- 500 buffers

- blocksize set to 512

- maxtransfer set to 7FFFFFFF

- mask set to FFFFFFFE

III - Fine Art of drive partitioning

===================================

A few things you must know :

- You'll get decent performance from any FS only with buffers >= 500

- You'll get decent performance from FFS only with blocksize >= 2048

- You'll get the best performance from SFS only with blocksize = 512

- SFS is way faster than FFS in its default (non-caching) behaviour,

but FFS is said to be safer.

- SFS + its diskcache.library is even faster.

- There is a cache plugin for FFS, and you can also set FFS in

writeback cache mode. Using both things make it waaaaay faster

(almost comparable to SFS). The FFS cache plugin is 'fs_cache_plugin',

you will find it somewhere on your OS4 CD, just 'run fs_cache_plugin DH0:'

for each FFS partition you wish to run with cache.

- The command I use to bench r/w perf is scsispeed, found in the

latest diskspeed archive from Aminet, and also in an OS4 native

form on the OS4 beta server (quick port by myself, dropped the

m68k CPU usage measurement code).